T16 SIM Resource Inventory v5

Interface Specification

Version 1.1.0 ● 09 Nov 2024

# Document Version Control

| Version | Date | Editor | Change Description |
| --- | --- | --- | --- |
| 1.0.0 | 09/11/2024 | Patriya Piyawiroj | Document Created |
| 1.1.0 | 27/11/2024 | Patriya Piyawiroj | Added sort and content-range query parameters |
| 1.1.1 | 12/12/2024 | Patriya Piyawiroj | Added default reservation. Separated List from Query API |

## Table of Contents

Document Version Control	1

Table of Contents	2

Introduction	3

Overview	4

SIM Diagram	4

TMF Entity Diagram	5

Interface Definition	6

Headers	6

Resource entity	6

SIM Resource Types	8

SIM Card	9

Resource Specification	9

Characteristics	9

Resource Relationships	9

eSIM Profile	9

Resource Specification	10

Characteristics	10

Resource Relationships	11

Logical SIM	11

The Logical SIM (logical resource) represents the attributes of a (physical) SIM card or eSIM that are provisioned	11

during mobile line order execution.	11

Resource Specification	11

Characteristics	11

Resource Relationships	12

Physical SIM	13

The Physical SIM (physical resource) represents the physical SIM card sold to the customer.	13

Resource Specification	13

Characteristics	13

Resource Relationships	13

IMSI	13

Resource Specification	13

Characteristics	13

Resource Relationships	14

MSISDN	14

Resource Specification	14

Characteristics	14

Resource Relationships	14

Sample Operations	15

Query Resource from the inventory:	15

List Resource from the inventory:	15

Update Resource from the inventory:	17

Common Sub-Resource/Entities	18

Note sub-resource	19

# Introduction

### Overview

This document lists all the services exposed by the Resource Inventory (TMF639 version 5.0) module.

All the services are available using the HTTP and CEDA format.

### SIM Diagram

White Boxes - Product

Green Boxes - Resources

Note that the SMDP profile (downloaded to device) is not included in the diagram because it is managed by the eSIM vendor.

This TMF structure stems from GSMA standards. These names are defined in TMFS008. https://projects.tmforum.org/wiki/display/PUB/TMFS008+Use+Case%3A+Service+and+Resource+Order+Management+for+Postpaid+Mobile+Subscribers+v3.3.0

### TMF Entity Diagram

Note that eSIM does not use serialNumber because it is not a physical resource.

ResourceStatus is also not used because the possible values are fixed across all resources.

# Interface Definition

The Resource Inventory Management API is defined in the

TMF639-Resource_Inventory_Management-v5.0.0.oas (yaml) Open API specification.

### Headers

Request Headers

These are HTTP headers used for logging and monitoring purposes. For Kafka headers, please see T16 CEDA.

| Variable | O/M | Type (Location) | Description |
| --- | --- | --- | --- |
| Content-Type | M | String | Indicates the body message format. This must be specified to tell the system to expect an array of JSON Patch operations. Example: Application/json-patch-query+json |
| useCaseStartTime | O | String | The datetime in which a particular event occurred. See T3 of each business usecase for what to send. Example: 2024-01-31T00:00:00+07:00 |
| user | M | Object | The user identifier |
| channel | M | String | The original front end system that initiated the request |

Response Headers

Some additional headers are returned for GET APIs.

| Variable | O/M | Type | Description |
| --- | --- | --- | --- |
| Content-Type | M | String | The content type. Example: Application/json |
| X-Total-Count | O | String | Returned for queries. The total number of matching resources. Can be used by the client to calculate the number of pages Example: 50 |
| Content-Range | O | String | Returned for queries. Describes the range of items returned in the response. Example shows the first 10 items from a total of 50 matched records. Indexed at 0 to match byte format. Example: items 0-9/50 Example (unsatisifiable range error): items */50 |

Status Codes

| HTTP Response Code | Description |
| --- | --- |
| 200 | Success |
| 206 | Success (Partial Content). The response body contains the requested range of data. |
| 403 | Forbidden |
| 404 | Data not found |
| 500 | Internal Server error |
| 501 | Not implemented. Unsupported filter. |

### Resource entity

| Field |  |  | O/M | Type (Length) | Description |
| --- | --- | --- | --- | --- | --- |
| id |  |  | M | String | Unique identifier of the resource |
| href |  |  | M | String | Hyperlink reference to the resource. Example: /resourceInventory/v5/eSimProfile/42 |
| name |  |  | M | String | Name of the resource Example: Sim Card |
| note |  |  | O | Array of Note | Note. Extra-information about the resource; e.g. useful to add extra information that could be useful for a human process. |
|  | id |  | O | String | The running number of the note within the array. |
|  | author |  | O | String | The person/user that created the note |
|  | date |  | O | String | The date the note was created |
|  | text |  | O | String | The text value of the note |
|  | @type |  | O | String | This defines the class name or sub-class Value: "Note" |
| resourceRelationship |  |  | O | Array of ResourceRelationship | A list of resources related to this resource. The fields inside are Mandatory only when the resourceRelationship object exists. |
|  | relationshipType |  | M/O | String | The type of related order, such as: [uses] if the resource relies on or consumes the functionality of another entity (e.g. a SIM “uses” an IMSI and MSISDN for its operations within the mobile network) |
|  | resource |  | M/O | ResourceRefOrValue | A reference to the related resource. |
|  |  | id | M/O | String | Unique identifier of the resource |
|  |  | href | M/O | String | Hyperlink reference |
|  |  | @referredType | M/O | String | The class name of the related resource Example: Imsi |
|  |  | @type | M/O | String | Type of the resource Example: ResourceRef |
|  | @type |  | M/O | String | This defines the class name or sub-class Extensible name Value: ResourceRelationship |
| resourceCharacteristic |  |  | M | Array of Characteristic | Describes a given characteristic of the resource through a name/value pair. The possible characteristics depends on the SIM Resource Types. (SIM Card will have difference characteristics from eSIM Profile) This is an abstract base class, the actual value is in one of the strongly-typed subclasses (see @type) |
|  | name |  | M | String | Name of the characteristic |
|  | value |  | M | String | Value of the characteristic |
|  | valueType |  | M | String | Data type of the value of the characteristic. A more descriptive/specific version of the type of the characteristic. This is custom defined and can help clients create custom logic to handle each characteristic differently. |
|  | @type |  | M | String | One of: StringCharacteristic, ObjectCharacteristic, FloatCharacteristic, BooleanCharacteristic, NumberCharacteristic, IntegerCharacteristic,StringArrayCharacteristic, ObjectArrayCharacteristic,BooleanArrayCharacteristic, NumberArrayCharacteristic, IntegerArrayCharacteristic... |
| relatedParty |  |  | M/O | Array of RelatedPartyRef | Parties such as customers and organizations related to this resource |
|  | role |  | M/O | String | Role of the related party in relation to this resource (e.g. owner, vendor) |
|  | partyOrPartyRole |  | M/O | PartyRef | A reference to the party information in party domain |
|  |  | id | M/O | String | The id of the party |
|  |  | name | M/O | String | The name of the party Example: AWN |
|  |  | href | M/O | String | Hyperlink reference Example: /partyManagement/v5/organization |
|  |  | @type | M/O | String | This defines the class name or sub-class Extensible name Value: PartyRef |
|  |  | @referredType | M/O | String | The class name of the related resource Example: Organization |
|  | @type |  | M/O |  | This defines the class name or sub-class Extensible name Value: ResourceRelationship |
| @baseType |  |  | O | String | When sub-classing, this defines the super- class. Value: "Resource" |
| @type |  |  | M | String | This defines the class name or sub-class Extensible name Value: "Imsi" |

# SIM Resource Types

## SIM Card

The SIM card compound resource consists of a physical SIM (selected during order capture or shipped to customer during product order processing) and a Logical SIM (which is provisioned during the order execution).

### Resource Specification

| Field | Description |
| --- | --- |
| name | SIM Card |
| category | Network Connectivity |
| description | The SIM card compound resource consists of a physical SIM (selected during order capture or shipped to customer during product order processing) and a Logical SIM (which is provisioned during the order execution). |
| version | 1.0 |
| path | /simCard |
| type | SimCard |

### Characteristics

Greyed out fields are duplicated from the associated resource relationship

| Name | Is Unique | Configurable | Extensible | Value Type | Description |
| --- | --- | --- | --- | --- | --- |
| status | No | Yes | N/A | String | Status Code. Possible values: • PendingPerso • Available • Cancelled • Expired • Write Off • Pending PortIn • Pending Prepaid Iden • Pending Preparation • Pending Recall • Pending Registration • PendingPerso • Prematch • Prepaid Iden • Registered • Reserved • Transfer Pre-paid Alternate name: status_code |
| simSerialNo | See Physical SIM |  |  |  |  |
| Iccid | See Physical SIM |  |  |  |  |
| regionCode | See IMSI |  |  |  |  |
| number | See MSISDN |  |  |  |  |
| networkCode | See Logical SIM |  |  |  |  |
| capacity | See Logical SIM |  |  |  |  |
| simVersion | See Logical SIM |  |  |  |  |

### Resource Relationships

| Name | RelationshipType | Role | Description |
| --- | --- | --- | --- |
| Logical SIM | uses | N/A | This resource provides attribute(s) that are provisioned during order execution |
| Physical SIM | uses | N/A | The resource represents the physical SIM |
| IMSI | uses | N/A | An International Mobile Subscriber Identity - a unique number used to identify a mobile subscriber in a cellular network. |
| MSISDN | uses | N/A | The mobile number associated with the SIM |

## eSIM Profile

The eSIM Profile logical resource is the provisioning view of the corresponding profile being downloaded to the end

user equipment

### Resource Specification

| Field | Description |
| --- | --- |
| name | eSIM Profile |
| category | Network Connectivity |
| description | The eSIM Profile logical resource is the provisioning view of the corresponding profile being downloaded to the end user equipment |
| version | 1.0 |
| path | /eSimProfile |
| type | ESimProfile |

### Characteristics

Greyed out fields are duplicated from the associated resource relationship

| Name | Is Unique | Configurable | Extensible | Value Type | Description |
| --- | --- | --- | --- | --- | --- |
| eid | Yes | Yes | N/A | String | [StringCharacteristic] “Embedded Identity Document” is a serial number corresponding to the eSIM installed in a connected device. The eSIM vendor contains this information and will send this value to AIS when the eSIM is installed on the user device. |
| smdpState | No | Yes | N/A | String | [StringCharacteristic] The state of the eSIM Profile. When the eSIM profile state is changed (e.g. eSIM Profile is downloaded by user) a notification is sent from the eSIM vendor to AIS, and the state is tracked in the esim table. Possible values: • Downloaded • Installed • Deleted • Enable • Disable Alternate name: esim.status_code |
| iccid | Yes | Yes | N/A | String | [StringCharacteristic] A 19 digit Integrated Circuit Card Identifier, uniquely identifies the SIM Card (UICC) on the network. For eSIMs, this number is maintained by the eSIM vendor, but its value is determined by AIS. When eSIMs are bought in batches from the vendor, AIS increments the running number of eSIMs kept as CMSE. Each number is prefixed with “896603” and then sent to the vendor to be set on the eSIM. |
| matchingId | Yes | Yes | N/A | String | [StringCharacteristic] A unique identifier used by the SMDP+ server to match a specific eSIM profile to the request made by the Local Profile Assistant (LPA) on the device. |
| simSerialNo | Yes | Yes | N/A | String | [StringCharacteristic] A 13 digit serial number tied to the physical SIM on procurement. For eSIMs, this a running number used to generate the iccid. Can not be changed. Alternate name: sim.serial_no |
| smdpAddress | No | Yes | N/A | String | [StringCharacteristic] The server URL of the vendor system managing eSIM profiles. This value is not stored in the database but is calculated by the app using the value of qrCodeInfo without the matchingId. |
| status | No | Yes | N/A | String | [StringCharacteristic] Status Code. Possible values: • PendingPerso • Available • Cancelled • Expired • Write Off • Pending PortIn • Pending Prepaid Iden • Pending Preparation • Pending Recall • Pending Registration • PendingPerso • Prematch • Prepaid Iden • Registered • Reserved • Transfer Pre-paid Alternate name: sim.status_code |
| qrCodeInfo | Yes | Yes | N/A | String | Contains QR Code information used to download the eSIM profile. Is acquired along with the eSIM when bought from vendor (ECP). Unique to eSIMs. Format: SMDPAddress concatenated with matchingId. |
| regionCode | See IMSI |  |  |  |  |
| number | See MSISDN |  |  |  |  |
| networkCode | See Logical SIM |  |  |  |  |
| capacity | See Logical SIM |  |  |  |  |
| simVersion | See Logical SIM |  |  |  |  |

### Resource Relationships

| Name | RelationshipType | Role | Description |
| --- | --- | --- | --- |
| Logical SIM | uses | N/A | This resource provides attribute(s) that are provisioned during order execution |
| IMSI | uses | N/A | An International Mobile Subscriber Identity - a unique number used to identify a mobile subscriber in a cellular network. |
| MSISDN | uses | N/A | The mobile number associated with the SIM |

### Related Parties

| role | Description |
| --- | --- |
| owner | The company that owns the sim Example: AWN |
| vendor | The eSIM vendor Example: ECP |

## Logical SIM

### The Logical SIM (logical resource) represents the attributes of a (physical) SIM card or eSIM that are provisioned

### during mobile line order execution.

### Resource Specification

| Field | Description |
| --- | --- |
| name | Logical SIM |
| category | Network Connectivity |
| description | The Logical SIM (logical resource) represents the attributes of a (physical) SIM card or eSIM that are provisioned during mobile line order execution. |
| version | 1.0 |
| path | /logicalSim |
| type | LogicalSim |

### Characteristics

| Name | Is Unique | Configurable | Extensible | Value Type | Description |
| --- | --- | --- | --- | --- | --- |
| accValue | No | Yes | N/A | String | [StringCharacteristic] The Access Control Class of the SIM |
| adm1 | No | Yes | N/A | String | [StringCharacteristic] The Administrative Key is a cryptographic key used for managing security-related operations on the SIM card, such as changing PIN codes, unlocking the card, or updating security settings |
| algorithm | No | Yes | N/A | String | [StringCharacteristic] The cryptographic algorithm used to generate and verify the EOPC. For example: Hash functions like SHA-256 (Secure Hash Algorithm 256-bit) or SHA-3 (Secure Hash Algorithm 3) for generating a unique hash value from input data. Symmetric encryption algorithms like AES (Advanced Encryption Standard) for encrypting and decrypting data securely. Asymmetric encryption algorithms like RSA (Rivest-Shamir-Adleman) for secure communication and key exchange between the device and the operator's servers. |
| capacity | No | Yes | N/A | String | [StringCharacteristic] SIM Capacity (e.g. "capacityCode" : "64k") |
| cardProfile | No | Yes | N/A | String | [StringCharacteristic] SIM Electrical Profile. A code which contains information on the card type, vendor, features (OTA, 5G, etc.). This is used to distinguish between different SIM Specifications and is manually assigned in by a user on SFF. |
| eki | No | Yes | N/A | String | [StringCharacteristic] The EKI (Encrypted Key Indicator) refers to a piece of information used in the process of securely exchanging encryption keys between the mobile device (such as a smartphone) and the mobile network operator's systems during over-the-air operations. |
| eopc | No | Yes | N/A | String | The Ephemeral Operator Provisioning Code is a temporary code that is used during the provisioning process of a SIM profile. It serves as a secure means for the device to authenticate and establish a connection with the mobile network operator's systems in order to download the necessary subscription information and configuration settings. |
| k4id | No | Yes | N/A | String | [StringCharacteristic] The Key for Identity (K4ID) is a cryptographic key used in SIM cards and eSIMs for identity authentication and security purposes. The K4ID is one of the keys stored on the SIM card or eSIM profile and is used to generate authentication tokens and signatures during various security-related operations. It plays a crucial role in verifying the identity of the mobile device (such as a smartphone |
| kic | No | Yes | N/A | String | [StringCharacteristic] The KIC (Key Encryption Key) is a cryptographic key used in SIM cards and eSIMs for securing communication between the mobile device and the mobile network operator's systems. Specifically, the KIC is used to encrypt data exchanged during authentication and over-the-air operations. |
| kycSupported | No | Yes | N/A | Boolean | [StringCharacteristic] A flag that indicates whether this SIM supports eKYC. If yes, the customer can use eKYC as a means to verify their information after prematching stage. Alternate name: sim.kyc_flag |
| networkCode | No | Yes | N/A | String | [StringCharacteristic] Also known as operator code. Used to identify the SIM’s telecom operator / service provider within a network. A numbered code e.g. 03 |
| networkName | No | Yes | N/A | String | [StringCharacteristic] The name of the operator/service provider. For example the 03 network code maps to “AWN”. Mapped from networkCode. |
| otaKey | No | Yes | N/A | String | [StringCharacteristic] The OTA (Over-The-Air) key, also known as an OTA cryptographic key, is a security feature used in mobile networks, particularly in the context of SIM cards and eSIMs. This key is used to securely authenticate and encrypt communication between the mobile device (such as a smartphone) and the mobile network operator's systems during over-the-air operations. |
| pin1 | No | Yes | N/A | String | [StringCharacteristic] This is the primary PIN associated with a SIM card. It is used to protect access to basic functionalities of the SIM card, such as making phone calls, sending SMS messages, and accessing mobile data services. Whenever the device is powered on or the SIM card is inserted, the user typically needs to enter the PIN1 to unlock the SIM card and gain access to these basic services |
| pin2 | No | Yes | N/A | String | [StringCharacteristic] This is a secondary PIN used to secure access to more sensitive or advanced functionalities on the SIM card. It is primarily used for accessing functions related to the SIM card's supplementary services, such as call forwarding, call barring, changing the PIN1, and accessing the fixed dialing list (FDN). PIN2 is required when performing operations that may have a greater impact on the subscriber's account or privacy |
| puk1 | No | Yes | N/A | String | [StringCharacteristic] PUK1 is used to unlock the SIM card when the primary PIN (PIN1) has been entered incorrectly multiple times. When a user enters an incorrect PIN1 several times in a row (usually three to ten times, depending on the mobile network operator's policy), the SIM card becomes locked, and the user is prompted to enter the PUK1 to unlock it. Entering the correct PUK1 allows the user to reset the PIN1 and regain access to the SIM card's basic functionalities. |
| puk2 | No | Yes | N/A | String | [StringCharacteristic] PUK2 is used to unlock supplementary services and functions on the SIM card when the secondary PIN (PIN2) has been entered incorrectly multiple times. Similar to PUK1, when a user enters an incorrect PIN2 several times in a row, the SIM card may become locked, requiring the user to enter the PUK2 to unlock access to functions such as call barring, call forwarding, and the fixed dialing list (FDN). |
| simProfile | No | Yes | N/A | String | [StringCharacteristic] A system profile used to categorise the SIM in to different capabilities |
| simVersion | No | Yes | N/A | String | [StringCharacteristic] Minimum support Version of SIM (e.g. "simVersion" : "3G") |

### Resource Relationships

None

## Physical SIM

### The Physical SIM (physical resource) represents the physical SIM card sold to the customer.

### Resource Specification

| Field | Description |
| --- | --- |
| name | Logical SIM |
| category | Network Connectivity |
| description | The Physical SIM (physical resource) represents the physical SIM card sold to the customer. |
| version | 1.0 |
| path | /physicalSim |
| type | PhysicalSim |

### Characteristics

| Name | Is Unique | Configurable | Extensible | Value Type | Description |
| --- | --- | --- | --- | --- | --- |
| iccid | Yes | Yes | N/A | String | [StringCharacteristic] A 19 digit Integrated Circuit Card Identifier, uniquely identifies the SIM Card (UICC) on the network. For physical SIMs, this value the simSerialNo prefixed with “896603” Example: 8966030000000000000 |
| simSerialNo | Yes | Yes | N/A | String | [StringCharacteristic] A 13 digit serial number tied to the physical SIM on procurement. Alternate name: sim.serial_no |

### Resource Relationships

None

## IMSI

International Mobile Subscriber Identity. A unique number used to identify a mobile subscriber in a cellular network. May come with or without the SIM depending on the vendor. For vendors such as Gemalto, the IMSI comes with the SIM. For vendors such as ECP, the IMSI life cycle must be managed by AIS separately.

### Resource Specification

| Field | Description |
| --- | --- |
| name | IMSI |
| category | Network Connectivity |
| description | International Mobile Subscriber Identity. A unique number used to identify a mobile subscriber in a cellular network. |
| version | 1.0 |
| path | /imsi |
| type | Imsi |

### Characteristics

| Name | Is Unique | Configurable | Extensible | Value Type | Description |
| --- | --- | --- | --- | --- | --- |
| imsi | Yes | Yes | N/A | String | [StringCharacteristic] Value of IMSI |
| dummyImsi | No | Yes | N/A | String | [StringCharacteristic] The dummy IMSI used when preprovisioning the SIM |
| region | No | Yes | N/A | String | [StringCharacteristic] Region. Defined as a 2 digit code e.g. C3 Alternate Name: parent_region |
| regionCode | No | Yes | N/A | String | [StringCharacteristic] A 5 digit code that specifies a region in Thailand. After Perso SIM when SIM is associated with an IMSI, the region of the SIM will depend on the parent region of the IMSI. Also stored in sim. Format: XX001 Where the first two letters represent the region (e.g. CW=Central West, NX=North). The first two numbers specifies a specific place within the region. The last number is 0 if is a postpaid SIM, and 9 or empty if is prepaid. The default value is prepaid. Mobile Region Code Remark: 1. mandatory when actionType = “requestESIM” 2. if client input this value, PGZINV will use it to random eSIM normal (Gemalto) 3. for dynamic eSIM (ECP), PGZINV cannot random by using regionCode because dynamic eSIM doesn’t has the imsi. 4. for actionType “randomImsiForESIM”, if client send only mobileNo (no regionCode), PGZINV will use mobileNo and simService send to SFF for random imsi. **client will send regionCode = 5 digits to randomImsiForESIM, PGZINV will pass this value to SFF ** |
| regionName | No | Yes | N/A | String | The spelled out name of the region. For example “Central” |

### Resource Relationships

May have a two way relationship back to the SIM with “usedBy”

## MSISDN

Mobile Station International Subscriber Directory Number. It is the unique phone number assigned to a mobile subscriber for identification and communication in a mobile network.

### Resource Specification

| Field | Description |
| --- | --- |
| name | MSISDN |
| category | Network Connectivity |
| description | Mobile Station International Subscriber Directory Number. It is the unique phone number assigned to a mobile subscriber for identification and communication in a mobile network. |
| version | 1.0 |
| path | /msisdn |
| type | Msisdn |

### Characteristics

| Name | Is Unique | Configurable | Extensible | Value Type | Description |
| --- | --- | --- | --- | --- | --- |
| number | Yes | Yes | N/A | String | The current active mobile number used by the SIM. |

### Resource Relationships

None

Sample Operations

### Query Resource:

Description: This operation gets information of a single eSIM and SIM. Used for singular queries only. The resource ID must be known, and a 404 error will be thrown if the resource ID is not found. As such, query parameters are not applicable and only one response object will be returned.

Methods: GET

Request URI (eSIM): /resourceInventory/v5/eSimProfile/{resourceId}

Request URI (physical SIM): /resourceInventory/v5/simCard/{resourceId}

#### Response Example:

{

"id": "be665c24-0c48-4bdd-8b69-e4790cee1d88",

"name": "eSim Profile ",

"note": [

{

"id": "1",

"text": "Insert by API simSerialNoPortIn because sim not perso by PHX",

"@type": "Note"

}

],

"resourceCharacteristic": [

{

"name": "simSerialNo",

"value": "2140115870164",

"valueType": "String",

"@type": "StringCharacteristic"

}

],

"resourceRelationship": [

{

"relationshipType": "uses",

"resource": {

"id": "8655",

"href": "resourceInventory/v5/resource/8655",

"@referredType": "Imsi",

"@type": "ResourceRef"

},

"@type": "ResourceRelationship"

}

]

"@type": "ESimProfile"

}

### List Resource:

Description: This operation gets information of an eSIM and SIM

Methods: GET

Request URI (eSIM): /resourceInventory/v5/eSimProfile

Request URI (physical SIM): /resourceInventory/v5/simCard

#### Query Parameters:

| Variable | O/M | Type | Description |
| --- | --- | --- | --- |
| fields | O | List of String (Query Parameter) | A TMF concept that filters the response to only include the fields requested. Accepts a list of values beginning with “resource”. Format: {field1},{field2}, …. Sample Value: resourceCharacteristic If left empty, all fields in database will be returned. Use to reduce payload size. Regardless of the filter, the “id”, “href” field will always be returned. |
| filter | O | String | Matches the documents with this JSON Patch filter. Unsupported filters will return 501 Not Implemented as per TMF630 Part 5 guidelines. Singular: ?filter=$.resourceCharacteristic[?(@.name=’simSerialNo’&&@.value=8966032140115870164)] Range: ?filter=$.resourceCharacteristic[?(@.name=’simSerialNo’&&@.value>=8966032140115870164&&@.value<8966032140115870164)] Multiple: ?filter=$.resourceCharacteristic[?(@.name=’simSerialNo’&& @.value in [1, 2, 3])] Supports the following names: • simSerialNo • iccid • status • networkCode • capacity • regionCode • prepMobileNo • simVersion JSON Path also accepts range (>=, >, <, <=) and equivalence (==, !=) queries. |
| note.text | O | String | Queries a SIM with the given remark Example: ?note.text=’Change SIM’ Alternate name: sim.remark |
| limit | O | String | Requested number of resources to be provided in response requested by client. If limit isn’t specified, default is 100, and max is 500. Exceeding max will not result in error, but only a maximum of 500 will be returned. Example: ?limit=100 |
| offset | O | String | Requested index for start of resources to be provided in response requested by client Example ?offset=10&limit=20 Gets 20 resources starting with the 10th |
| expand | O | String | A list of fields to expand. Used to limit expands. If this is not specified, all fields and the given depth will be expanded. Example: ?expand=resourceInventory.resourceRelationship |
| depth | O | String | The number of depths to expand. If depth is not specified, then the default value will depend on the expand parameter. If expand parameter is set, then depth is 1. If not set, then 0. Examples: • ?depth=0, none of the ids and hrefs will be expanded • ?depth=1, Replaces all hrefs at the resource level • ?depth=2, Replaces all hrefs in variables of resource (e.g. resource.resourceRelationship • ?depth=3, (e.g. resource.resourceRelationship.resource) • depth of 4 and more will throw 501 Unsupported error |
| sort |  |  | Sorts the results by a field. Supports JSON Path sorting for characteristics Example: • ?sort=id (Sort the result set on id) • ?sort=-id (Sort the result set on id in descending order) • ?sort=resourceCharacteristic[?(@.name==”simSerialNo”)].value • ?sort=id,resourceCharacteristic[?(@.name==”createdDate”)].value (Sort by multiple parameters) |

#### Response Example:

[

{

"id": "be665c24-0c48-4bdd-8b69-e4790cee1d88",

"name": "eSim Profile ",

"note": [

{

"id": "1",

"text": "Insert by API simSerialNoPortIn because sim not perso by PHX",

"@type": "Note"

}

],

"resourceCharacteristic": [

{

"name": "simSerialNo",

"value": "2140115870164",

"valueType": "String",

"@type": "StringCharacteristic"

}

],

"resourceRelationship": [

{

"relationshipType": "uses",

"resource": {

"id": "8655",

"href": "resourceInventory/v5/resource/8655",

"@referredType": "Imsi",

"@type": "ResourceRef"

},

"@type": "ResourceRelationship"

}

]

"@type": "ESimProfile"

}

]

### Update Resource:

Description: This operation updates a SIM

Kafka Topic (Consume eSIM): esb.prd.updateEsimProfile

Kafka Topic (Produce eSIM): esb.prd.esimProfileUpdated

Kafka Topic (Consume sim card): esb.prd.updateSimCard

Kafka Topic (Produce sim card): esb.prd.simCardUpdated

##### Request Example:

JavaScript Object Notation (JSON) [RFC4627] is a common format for the exchange and storage of structured data. HTTP PATCH [RFC5789] extends the Hypertext Transfer Protocol (HTTP) [RFC2616] with a method to perform partial modifications to resources.

Please refer to this example for patching resource characteristics:

[{

"op": "replace",

"path": "resourceCharacteristic/value?resourceCharacteristic.name=status",

"value": "Available"

},{

"op": "replace",

"path": "resourceCharacteristic/value?resourceCharacteristic.name=smdpState",

"value": "Installed"

}, {

"op": "replace",

"path": "relatedParty/partyOrPartyRole/name?relatedParty.role=owner",

"value": "AWN"

},

{

"op": "replace",

"path": "relatedParty/partyOrPartyRole/name?relatedParty.role=vendor",

"value": "ECP"

},

]

And refer to this example for adding/removing a resource relationship:

,[{

"op": "add",

"path": "resourceRelationship ",

"value": {

“relationshipType”: “uses”,

“resource”: {

“id”: “”,

“@type”: “MSISDN”

}

}

}]

##### Response Example:

[{

"op": "replace",

"path": "resourceCharacteristic/value?resourceCharacteristic.name=status",

"value": "Reserved"

},{

"op": "add",

"path": "resourceRelationship ",

"value": {

“relationshipType”: “uses”,

“resource”: {

“id”: “”,

“@type”: “IMSI”

}

}

},{

"op": "add",

"path": "resourceRelationship ",

"value": {

“relationshipType”: “uses”,

“resource”: {

“id”: “”,

“@type”: “MSISDN”

}

}

}]

### Create Resource:

Description: This operation creates a SIM

Kafka Topic (Consume eSIM): esb.prd.createEsimProfile

Kafka Topic (Produce eSIM): esb.prd.esimProfileCreated

Kafka Topic (Consume sim card): esb.prd.createSimCard

Kafka Topic (Produce sim card): esb.prd.simCardCreated

##### Request Example:

{

"name": "eSim Profile ",

"note": [

{

"id": "1",

"text": "Insert by API simSerialNoPortIn because sim not perso by PHX",

"@type": "Note"

}

],

"resourceCharacteristic": [

{

"name": "simSerialNo",

"value": "2140115870164",

"valueType": "String",

"@type": "StringCharacteristic"

}

],

}

##### Response Example:

{

"id": "be665c24-0c48-4bdd-8b69-e4790cee1d88",

"name": "eSim Profile ",

"note": [

{

"id": "1",

"text": "Insert by API simSerialNoPortIn because sim not perso by PHX",

"@type": "Note"

}

],

"resourceCharacteristic": [

{

"name": "simSerialNo",

"value": "2140115870164",

"valueType": "String",

"@type": "StringCharacteristic"

}

]

"@type": "ESimProfile"

}

### Delete Resource:

Description: Hard deletes a resource

Methods: DELETE

Request URI (eSIM): /resourceInventory/v5/eSimProfile/{resourceId}

Request URI (physical SIM): /resourceInventory/v5/simCard/{resourceId}

#### Response Example:

200 Success

Common Sub-Resource/Entities

### Note sub-resource

| Field | O/M | Type (Length) | Description |
| --- | --- | --- | --- |
| id | O | String | The running number of the note within the array. |
| author | O | String | The person/user that created the note |
| date | O | String | The date the note was created Format: date-time FRC3339 |
| text | O | String | The text value of the note |
| @type | O | String | This defines the class name or sub-class Value: "Note" |